Traffic Management Branch

RTA, NSW

Procedure:	     Traffic Route Lighting Subsidy Scheme

Initiation and Eligibility of Proposal

Document Ref No.:              RTA-TMB-QP-101

| Issue No | Details | Prepared by | Approved by | Date |
| --- | --- | --- | --- | --- |
| 1 | Initial issue | R Walsh | GM, TM | 01/10/97 |
| 2 | Amended in accordance with Standard AS/NZS 1158.1.1 – 1997, and incorporating extracts from the former Works Policy Manual RTA references amended | R Walsh | GM, TM | 23/03/02 |

MSWord file:    /tmp/in/input.doc

Traffic Route lighting Subsidy Scheme

Initiation and Eligibility of Proposal

1.0	Purpose

1.1	This procedure defines the methods required for initiating the installation or upgrading of Traffic Route Lighting, and

1.2	The methods by which such an installation or upgrading is determined to be eligible for subsidy under the Traffic Route Lighting Subsidy Scheme.

2.0	Background

2.1	Funding the provision and maintenance of street lighting is generally a matter for Councils.

2.2	Provision is made in Section 10Y of the Traffic Act 1909 for financial assistance to Local Government Councils (Councils) to support the provision of an adequate standard of lighting on important traffic routes under the Traffic Route Lighting Subsidy Scheme (TRLSS).

2.3	This lighting is considered eligible for subsidy subject to:

2.3.1.	the scheme being satisfactorily assessed, using Clauses 4.1 to 4.6, Part 12 of AUSTROADS Guide to Traffic Engineering Practice - Roadway Lighting, as guidelines.  It should be noted that this document reflects the former SAA Public Lighting Code AS1158.1-1986, which has since been superseded by the Road Lighting Standard AS/NZS 1158.1.1–1997.  Users should consult AS/NZS 1158.1.1 where the Austroads Guide makes reference to AS1158.1.

2.3.2.	being installed as Category V3 lighting as specified in AS/NZS 1158.1.1-1997.

2.4	The extent of a lighting installation may be a length of road, a roundabout or channelised intersection, or the flood lighting of a marked pedestrian crossing.  In the case of a marked pedestrian crossing located on a route not lit to the relevant Standard, it is usually required, at least, that the immediate approaches to the crossing be lit to the required standard before accepting the flood lighting for subsidy.

2.5	The RTA is responsible for the delivery of the TRLSS, with eligibility for inclusion in the TRLSS determined by the General Manager, Traffic Management (GM, TM).

2.6	The level of assistance available to Councils is determined annually by the Minister for Roads.

2.7	Special installations may be requested of Councils by the RTA for works such as motorway construction. Details for the possible payment of subsidy in these instances are detailed in Section 2.7 of Procedure RTA-TMB-QP-102.

2.8	The RTA meets the cost of lighting freeways, controlled access roads, certain facilities, such as tunnels, major bridges and some exceptional channelised intersections in rural areas. Funding for these works are excluded from the TRLSS.

3.0	definitions

3.1	“Traffic route” means:

(a)	a state road or regional road; or

(b)	a public road (other than a state road or regional road) that the RTA has, by reason of the volume of vehicular or pedestrian traffic carried on it, determined requires lighting to a standard approved by the RTA.

4.0	Responsibilities

4.1	Council:

4.1.1	Identifies which schemes may be eligible for subsidy.

4.1.2	Prepares a submission based on the warranting guidelines.

4.1.3	Forwards the submission to the RTA Regional Office with which the Council liaises.

4.1.4	On receipt of formal RTA approval, requests the Electricity Distributor (or other approved agency) to prepare a design for lighting to the approved standard.

4.1.5	Approves the scheme design and arranges for its installation or upgrading by the Electricity Distributor and requests that the Scheme be categorised as TRLSS for invoicing purposes.

4.1.6	Advises the RTA’s Regional Manager or the General Manager, Sydney Client Services (GM, SCS) when the installation has been completed.

4.2	The RTA’s Regional Manager or the GM, SCS:

4.2.1	Forwards the Council’s submission, together with a recommendation to the GM, TM for formal approval.

4.2.2	Notifies the Council of the decision regarding eligibility of the proposal for inclusion in the Scheme.

4.2.3	Notifies the GM, TM when the Scheme has been completed.

4.3	The Electricity Distributor:

4.3.1	Designs and/or installs the Scheme following Council’s formal request.

4.3.2	Maintains an inventory for invoicing purposes.

4.3.3	Notifies the Council when the Scheme has been completed.

4.4	The GM, TM approves all installations eligible for subsidy.

5.0	Procedure details

6.0	ASSOCIATED DOCUMENTS

| Document No. | Title | Issue No. | Issue Date |
| --- | --- | --- | --- |
| RTA-TMB-QP-102 | Procedure Traffic Route Lighting Subsidy Scheme - Subsidy Payments to Councils | 2 | 23/03/02 |
|  | AUSTROADS Guide to Traffic Engineering Practice. Part 12 - Roadway Lighting |  | 1988 |
| AS/NZS 1158.1.1 | –Road Lighting Part 1.1 Vehicular traffic (Category V) lighting - Performance and installation design requirements |  | 1997 |